| MSA Unit 136 | QCF Ref: XXXXX |  |
| --- | --- | --- |
| Title: | Control Roll On – Roll Off Operations On Domestic Vessels |  |
| Level: | 3 |  |
| Credit value: | 2 |  |
| Learning outcomes - The learner will: |  | Assessment criteria - The learner can: |
| 1. Know the regulatory framework within which Roll On–Roll Off (Ro-Ro) operations are carried out |  | 1.1 Explain the content of Maritime & Coastguard Agency (MCA) ‘M’ notices covering Roll On–Roll Off (Ro-Ro) operations. 1.2 Explain the requirements of Safety Management Systems as they relate to Ro-Ro operations 1.3 Explain the sections of the Maritime & Coastguard Agency (MCA) Code of Safe Working Practices for Merchant Seamen that relate to Ro-Ro operations 1.4 Explain the content of the MCA Code of Ro-Ro Ships Stowage and Securing of Vehicles 1.5 Explain the content of Cargo Securing Manuals |
| - 2. Understand the specific stability considerations applying to Ro-Ro operations |  | - 2.1 Explain the information contained in a ship’s stability book - 2.2 Explain the effects on stability during and after loading and discharging of cargo - 2.3 Explain the effect of free surface in fuel and water tanks on stability at all stages of loading and discharging - 2.4 Explain the effect on stability of free surface water on vehicle decks - 2.5 Explain the effect of point and axle weight loading on stability on vehicle decks and ramps - 2.6 Explain the stability calculations which must be completed before sailing - 2.7 Explain the effect of flooding and damage stability factors |
| - 3. Know the systems for opening and closing bow and stern doors, other hull openings, mezzanine decks, and vehicle ramps |  | - 3.1 Explain how to operate bow and stern doors, other hull openings, mezzanine decks, and vehicle ramps - 3.2 Explain the checks to be carried out on watertight seals and securing devices - 3.3 Explain the visual and audible alarms relating to vehicle deck operations |
| - 4. Know how to control vehicle deck operations |  | - 4.1 Explain how to plan the loading and discharging of vehicles and foot passengers including: - briefing crew members on the intended operation - checking vehicles presented for shipment are suitable for carriage, including those carrying dangerous goods and livestock - separation of vehicles in different categories - distribution of vehicles on vehicle decks taking into account vehicle categories, weights, stability, trim and draft - maintaining adequate spacing between vehicles to allow passenger and crew access - separation of foot passengers and vehicles when both use the same ramp - 4.2 Explain safety procedures to be followed for vehicle deck operations - 4.3 Explain the audible and visual alarms in use during vehicle deck operations - 4.4 Explain the requirements and procedures for counting passengers - 4.5 Explain the personal protective equipment which must be worn by crew engaged in vehicle deck operations - 4.6 Explain how to maintain effective communications and give directions to crew members throughout the operation - 4.7 Explain the need for crew members to be able to give clear directions and instructions to vehicles and passengers - 4.8 Explain the reports and communications required between the vehicle deck, bridge, other departments, and passenger decks |
| 5. Know the requirements for the carriage of dangerous goods |  | - 5.1 Explain the content of the International Maritime Dangerous Goods Code (IMDG Code) - 5.2 Explain the purpose of a Document of Compliance for the Carriage of Dangerous Goods on a vessel - 5.3 Explain how to implement the provisions of the IMDG Code - 5.4 Explain where to obtain further advice on dangerous goods presented for shipment - 5.5 Explain which classes of dangerous goods are not permitted to be carried on board passenger vessels - 5.6 Explain emergency procedures to follow in the event of any emergency arise involving dangerous goods |
| 6. Know how to secure vehicles |  | - 6.1 Explain the different securing methods available - 6.2 Explain the different securing points which may be found on vehicles - 6.3 Explain the different securing points and devices which may be found on a vessel - 6.4 Explain how different items of equipment are used to secure a vehicle - 6.5 Explain how a vehicle’s own fittings may be used to assist in securing the vehicle - 6.6 Explain the maintenance of securing equipment - 6.7 Explain how to assess the number and level of vehicle securing devices to be used taking account of forecast sea and weather conditions, and vessel condition and limitations |
| 7. Know how to control vehicle deck atmosphere |  | # 7.1 Explain the requirements for vehicle deck ventilation # 7.2 Explain the operation of ventilation machinery # 7.3 Explain the different levels of ventilation which may be required during loading and discharging, and on passage # 7.4 Explain how the atmosphere can be tested |

| Additional information |  |
| --- | --- |
| Additional information about the unit | This unit is designed for study by those working towards meeting the requirements for a BML (Boatmaster Licence) Ro-Ro Operations Endorsement |
| Unit aim(s) | The aim of the unit is to provide the knowledge underpinning proficiency for controlling Ro-Ro operations on domestic vessels, including the requirements for a BML Ro-Ro Operations Endorsement |
| Unit expiry date |  |
| Details of the relationship between the unit and relevant national occupational standards (if appropriate) | MSA Maritime NOS 2012: A01, B14, B35 |
| Details of the relationship between the unit and other standards or curricula (if appropriate) | MCA syllabus for the BML Ro-Ro Operations Endorsement |
| Assessment requirements specified by a sector or regulatory body (if appropriate) | Knowledge will be tested either in writing or orally, (and if the latter subsequently recorded). |
| Endorsement of the unit by a sector or other appropriate body (if required) | Maritime Skills Alliance Maritime & Coastguard Agency |
| Location of the unit within the subject/sector classification system | Transportation |
| Name of the organisation submitting the unit | SQA, for the Maritime Skills Alliance |
| Availability for use |  |
| Availability for delivery |  |
| Guided Learning Hours | 20 |